INCLUDE SECTION 107 ON ALL PROJECTS

$$107.00A

Section 107. — LEGAL RELATIONS AND RESPONSIBILITY TO THE PUBLIC

Revised 16 June 2026

INCLUDE THE FOLLOWING ON ALL PROJECTS (FAR CLAUSES 52-222-26 AND 52-222-27 HAVE BEEN REMOVED)

##107.03A

107.03.  Delete Subsections (a) and (f).

INCLUDE THE FOLLOWING ON ALL PROJECTS

##107.10A

107.10(b).  Delete the first and second paragraphs and substitute the following:

Submit a hazardous spill plan at least 2 days before starting work, as required by Federal, State, or local regulations. Where the project meets the applicability requirements of 40 CFR 112, submit a “Spill Prevention, Control, and Countermeasure (SPCC) Plan”.

Describe preventative measures including the location of refueling and storage facilities and the handling of hazardous material. Describe actions to be taken in case of a spill. Identify personnel, equipment, and resources available to address spill prevention and response.