INCLUDE SECTION 629 AS NEEDED BASED ON CLAUSE INSTRUCTIONS

$$629.00A

Section 629. — ROLLED EROSION CONTROL PRODUCTS AND CELLULAR CONFINEMENT SYSTEMS

Revised 15 December 2025

INCLUDE CLAUSES ##629.06A AND ##629.06B ON ALL PROJECTS WITH RECP WORK

##629.06A

629.06(a).  Delete Subsection (3) and substitute the following:

(3) Check slot. Install two rows of fasteners 4 inches apart on 4-inch centers across the top edge of the RECP. Drive fasteners flush with soil surface.

##629.06B

629.06(b).  Delete the second paragraph and substitute the following:

Securely fasten RECP to the soil by installing fasteners at a minimum rate of 2 per square yard. Significantly higher anchor rates may be necessary in sandy, loose, or wet soils and in severe applications.

INCLUDE THE FOLLOWING ON ALL PROJECTS WITH CCS WORK

##629.07A

629.07.  Delete the first paragraph and substitute the following:

Excavate to the depth of the cellular confinement system and smooth and compact the slope. Install the top of the system flush or lower than the adjacent slope. Expand the geocell down the slope. Connect adjacent geocell sections with fasteners in every other cell.