UTILITY DATA QUALITY LEVEL

Project Number and Name: ___________________________________

Utility Data Quality Level (Mark A, B, C or D)

- Quality Level D:  These tasks are completed:

  - Research company name and contacts, as-builts/maps, digital data, and permit/agreement documents.

  - Review mapping, photos/imagery, and documents to determine if additional utility research is needed.

- Quality Level C:  These tasks (inclusive of QL D) are completed:

  - Map surface utility features - poles, pedestals, valves, manholes, overhead wires, etc.

  - Incorporate utility information and utility mapping data, including types and dimensions, materials, wire heights, easements, etc., into digital base map.

  - Review digital base map & identify data deficiencies and discrepancies.

  - Conduct additional research to resolve data deficiencies and discrepancies.

- Quality Level B:  These tasks (inclusive of QL D & C) are completed:

  - Perform underground lines detection & marking using acceptable methods.

  - Survey marked underground utilities & incorporate data into digital base map.

  - Resolve data deficiencies and discrepancies in the digital base map.

- Quality Level A:  These tasks (inclusive of QL D, C, & B) are completed:

  - Determine need for subsurface utilities investigations.

  - Select boring locations and method(s) of excavation.

  - Notify utility owner(s) and coordinate inspection or other on-site assistance as required.

  - Notify landowner(s) and obtain right of entry to perform work.

  - Perform excavations and survey locations of subsurface utilities; include depths, types and dimensions, materials, etc.

  - Restore excavation sites to pre-existing conditions.

  - Incorporate surveyed subsurface utility data into the digital base map.

This document conforms to the “Standard Guideline for Investigating and Documenting Existing Utilities, ASCE/UESI/CI 38-22”

Signature (Survey/ROW Specialist):                                                    Date: ______