US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 211 Roadway Obliteration

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Roadway Obliteration Description |  |
|  | 1. If method 1 was roadway obliterated to approximate original ground contours while keeping material within original construction limits? (211.01a) |  |
|  | 2. If method 1 were slopes finished to provide gradual transitions in slope adjustments without noticeable breaks? (211.01a) |  |
|  | 3. If method 2 were roads closed by filling ditches and out sloping the roadbed to drain? (211.01(b)) |  |
|  | 4. If method 2 was embankment material removed and sloped at localized drainages to restore the natural drainage patterns? (211.01b) |  |
|  | 5. If method 2 were all ruts and low spots that could hold water eliminated? (211.01b) |  |
|  | Construction Requirements for Rigid material |  |
|  | 6. If non-asphalt material (such as concrete pavement, curbs, gutters, sidewalks or other non-asphalt material) was material broken into pieces with maximum dimensions of 12 inches when burying according to subsection 203.05c? (211.02(a)(1)) |  |
|  | 7. As an alternative to breaking, was rigid material removed from the project and disposed of according to subsection 203.05a? (211.02(a)(1)) |  |
|  | 8. If asphalt material, was material disposed of in accordance with state and local regulations? (211.02(a)(2)) |  |
|  | 9. If asphalt material disposal required a permit was a copy submitted? (211.02a) |  |
|  | Construction Requirements for Non-Rigid material |  |
|  | 10. If non-asphalt material, was the gravel, crushed stone or other non-rigid surface, base and sub base material scarified and ripped then mixed with underlying soil and buried under at least 12 inches of soil? (211.02(b)(1)) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 11. Was asphalt-contaminated material disposed of according to subsection 211.02(a)(2)? (211.02(b)(2)) |  |
|  | Waterbars and Barriers |  |
|  | 12. Were waterbars and barriers constructed to prevent vehicle access? (211.03) |  |
|  | 13. |  |
|  | 14. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: