US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 404 Minor Hot Asphalt Concrete

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | Composition of Mix |  |
|  | 1A1. Was a Job-Mix Formula submitted and approved? (404.02) |  |
|  | 1A2. Was the strength, quality, and gradation specifications of the asphalt concrete mix submitted? (404.02) |  |
|  | 1A3. Were laboratory test reports that demonstrate the properties of the aggregates, asphalt binder, additives, and mix meeting Federal or State agency specifications submitted? (404.02) |  |
|  | Surface Preparation |  |
|  | 1B. Was the surface prepared according to Section 209 or Subsection 410.05 as applicable? (404.03) |  |
|  | Weather Limitations |  |
|  | 2. Was asphalt concrete placed on a dry, unfrozen surface when the temperature in the shade was at least 35F and rising? (404.04) |  |
|  | Hauling |  |
|  | 3. Was the asphalt mix hauled using vehicles conforming to Subsection 401.11? (404.05) |  |
|  | Placing |  |
|  | 4. Was mix placed with a mechanical paver? (404.06) |  |
|  | 5. Roadway paving lifts were not placed thicker than 4 inches? (404.06) |  |
|  | 6. In areas where mechanical spreading and finishing was impractical, was spreading and finishing of each course accomplished by hand raking, screeding, or other approved methods? (404.06) |  |
|  | 7. Was the surface constructed in a uniform texture and cross-section? (404.06) |  |
|  | 8. Were the joints constructed according to Subsection 401.15? (404.06) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | Compacting (Roadway Paving) |  |
|  | 9. Was mix compacted to a minimum of 90 percent of maximum specific gravity (density)?(404.07a) |  |
|  | 10. Was compaction completed before the mix temperature fell below 160F? (404.07a) |  |
|  | 11. Was density determined by nuclear gauge? (404.07a) |  |
|  | Compacting (Non-Roadway Paving) |  |
|  | 12. Was compaction accomplished by rolling with a hand-operated roller weighing at least 300 pounds or with a small power roller? (404.07b) |  |
|  | 13. In areas that were not accessible to rollers was compaction accomplished by another approved method? (404.07b) |  |
|  | Pavement Smoothness |  |
|  | 14. Was a 10-foot metal straightedge used to measure at right angles and parallel to the centerline looking for deviations in excess of ¼ inch in 10 feet between any two contacts of the straightedge with the surface? (404.08) |  |
|  | 15. Were defective areas corrected using approved methods? (404.08) |  |
|  | 16. |  |
|  | 17. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: