US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 634 Permanent Pavement Markings

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. Where existing and final pavement marking locations were identical, were the limits of all existing pavement markings (no-passing zones, edge stripes, etc.) staked before any pavement work? (634.03) |  |
|  | 2. Upon completion of the final surface course, were line limits established for the new pavement for approval before marking? (634.03) |  |
|  | 3. Were the markings established according to the MUTCD? (634.03) |  |
|  | 4. In curve widening areas, were the pavement edge markings applied at the edge of the traveled way and the centerline markings midway between the pavement lines? (634.03) |  |
|  | 5. Were all loose particles, dirt, tar, grease and other deleterious material removed from the surface to be marked? (634.03) |  |
|  | 6. Where markings are placed on rigid pavement less than 1 year old, was the pavement cleaned of all residue and curing compounds? (634.03) |  |
|  | 7. Were temporary pavement markings removed the same day permanent pavement markings were applied? (634.03) |  |
|  | 8. Were markings applied to a clean, dry surface according to the MUTCD? (634.03) |  |
|  | 9. Was a written copy of the marking manufacturer’s recommendation furnished at least 7 days before applying pavement markings? (634.03) |  |
|  | 10. Was a field demonstration required to verify the adequacy of recommendation? (634.03) |  |
|  | 11. Was marking material shipped in appropriate containers plainly marked with the following information, as appropriate, for the material being furnished: Manufacturer’s name and address, Name of product, Lot/Batch numbers, Color, Net mass and volume of contents, State of manufacture, Date of expiration, Statements of contents, Mixing proportions and instructions and Safety information? (634.03 a thru j) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 12. Were the pavement markings applied in the direction of traffic according to the manufacturer’s recommendations? (634.03) |  |
|  | 13. Were all markings applied to provide a clean-cut, uniform and workmanlike appearance by day and night? (634.03) |  |
|  | 14. Were lines 4 inches wide? (634.03) |  |
|  | 15. Were broken lines 10 feet long with 30-foot gaps? (634.03) |  |
|  | 16. Were dotted lines 2 feet long with 4-foot gaps? (634.03) |  |
|  | 17. Were double lines separated with a 4-inch space? (634.03) |  |
|  | 18. Were marked areas protected from traffic until the markings were dried to no-tracking condition? (634.03) |  |
|  | 19. Were all tracking marks, spilled marking material, markings in unauthorized areas and defective markings removed? (634.03) |  |
|  | 20. Were all conflicting pavement markings removed according to Subsection 635.13? (634.03) |  |
|  | Waterborne Traffic Paint (Type B and C) |  |
|  | 21. Was the paint applied when the pavement and air temperatures were above 50ºF? (634.05) |  |
|  | 22. Was paint sprayed at 15 mil minimum wet film thickness before glass beads or at a rate of 107 square feet per gallon? (634.05) |  |
|  | 23. Were type 1 glass beads immediately applied on the paint at a minimum rate of 6 pounds per gallon of paint? (634.05a) |  |
|  | 24. On new asphalt pavements or new asphalt surface treatments were two coats applied? (634.05) |  |
|  | 25. Was each coat applied at 210 square feet per gallon? (634.05) |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: