US DEPARTMENT OF TRANSPORTATION – FEDERAL HIGHWAY ADMINISTRATION

Construction Inspection Checklist

Section 636 Signal, Lighting and Electrical Systems

| Project Name: | Project No.: |
| --- | --- |
| Date: | Weather: |
| Contractor: | Subcontractor: |
| Inspector: | Location/Station: |
| Description of work being inspected: |  |

| Conformance | CHECKS (characteristics) |  |
| --- | --- | --- |
| Yes No N.A. |  |  |
|  | General |  |
|  | 1. At the preconstruction conference, was a certified cost breakdown of items involved in the lump sum submitted for use in making progress payments and price adjustments? (636.04) |  |
|  | 2. Was a list of proposed equipment and material, along with manufacturer’s name, size and identification number of each item, submitted fifteen days before installation? (636.04) |  |
|  | 3. Was a supplemental list with scale drawings, catalog cuts, and wiring diagrams showing locations and details of equipment and wiring provided? (636.04) |  |
|  | 4. Did the CO establish the exact locations of the system? (636.04) |  |
|  | 5. Were structures and obstructions removed according to Section 203? (636.04) |  |
|  | 6. Was all material acceptable for reuse salvaged? (636.04) |  |
|  | 7. Was excavation and backfill according to Section 209? (636.04) |  |
|  | 8. Was concrete construction according to Section 601? (636.04) |  |
|  | 9. Where roadways were to remain open to traffic and existing systems were modified, were the existing systems maintained in operation until final connection to the modified circuit to minimize traffic disruption? (636.04) |  |
|  | Conduit |  |
|  | 10. Was conduit cut so the ends were smooth? (636.05) |  |
|  | 11. Were the conduit sections connected with couplings that butted the ends of both conduits squarely against each other inside the couplings? (636.05) |  |
|  | 12. Were metal expansion and deflection fittings provided where conduit crossed a structural expansion joint? (636.05) |  |
| Conformance | CHECKS (characteristics) |  |
| Yes No N.A. |  |  |
|  | 13. Were conduits installed continuous between outlets with a minimum of couplings to permit pulling conductors? (636.05) |  |
|  | 14. Was conduit terminated with bell fittings or bushings? (636.05) |  |
|  | 15. Were pull wires furnished for conduits designated for future cable installation? (636.05) |  |
|  | 16. Were crushed, deformed or damaged conduit removed and replaced? (636.05) |  |
|  | 17. Did conduits remain clean and dry? (636.05) |  |
|  | 18. Were conduit ends protected with plugs, caps or fittings? (636.05) |  |
|  | 19. Were pull boxes sized to provide for termination of the conduit and connection of the conductor? (636.05) |  |
|  | 20. |  |
|  | 21. |  |
|  | 22. |  |
| Percent Conformance Calculations |  | YesYes + No(100)= (100)= % Conformance |

Comments: